### 🧠 How MIPS Calculates a Jump Address (Easy Version)

In MIPS, when you use a jump instruction (like j label), the processor doesn’t store the full 32-bit memory address of where to go. Instead, it uses a 26-bit number inside the instruction. Here’s how the CPU figures out the real address:

#### 🔁 Step-by-Step:

- Start from the next instruction:
The CPU always knows where it is—this is the Program Counter (PC).
But since the PC points to the current instruction, the next one is at PC + 4 (because each instruction is 4 bytes long).

- Keep the top 4 bits of (PC + 4):
Memory addresses in MIPS are 32 bits long.
To stay in the same “region” of memory (like the same 256 MB block), we reuse the top 4 bits from PC + 4.

- Take the 26-bit number from the jump instruction:
This is like a “local” address—it tells you where to go within that region.

- Multiply that 26-bit number by 4 (or shift left by 2 bits):
Why? Because every instruction starts at a multiple of 4 (they’re 4 bytes long).
So the real address must end in 00—shifting left adds those two zeros.

- Put it all together:
Final address =
[Top 4 bits from (PC + 4)] + [26-bit number × 4]

In other words:

Jump Address = { (PC + 4)[31:28], instruction[25:0] << 2 }

🎯 Goal:

Show how the final jump address is built using the rule:

Final Address = [Upper 4 bits of (PC + 4)] + [26-bit target from instruction, shifted left by 2]

📌 Given in the Example:

- Current PC points to a jump instruction at address: 0x1000_001C
→ So the next instruction would be at PC + 4 = 0x1000_0020

- The 26-bit target field inside the jump instruction is: 0x001000
(This is just the raw bits stored in the instruction—not a full address yet!)

🔍 Step 1: Get PC + 4

PC + 4 = 0x1000_0020

In binary (32 bits):

0001 0000 0000 0000 0000 0000 0010 0000

↑↑↑↑

These are bits [31:28] → the **upper 4 bits** = `0001`

We’ll keep these 4 bits to stay in the same 256 MB memory region.

🔍 Step 2: Take the 26-bit target from the instruction

The instruction contains:
target = 0x001000 (hex)
In binary, that’s a 26-bit number (we can pad with leading zeros if needed):

0000 0000 0001 0000 0000 0000   ← 26 bits

🔍 Step 3: Shift left by 2 (multiply by 4)

Why? Because all instructions are aligned to 4-byte boundaries → last 2 bits must be 00.

Shift left by 2:

0000 0000 0001 0000 0000 0000 00   ← now 28 bits

In hex, this is:

0x001000 << 2 = 0x004000

🔍 Step 4: Combine the parts

Now glue the upper 4 bits from PC+4 (0001) to the front of this 28-bit shifted value:

Upper 4 bits:      0001

Shifted target:    0000 0000 0100 0000 0000 0000 0000

(that’s 0x004000 in 28 bits)

Full 32-bit address:

0001 0000 0000 0100 0000 0000 0000 0000

Convert that to hex:

- Group by 4 bits: 0001 0000 0000 0100 0000 0000 0000 0000

- Hex: 0x10040000

✅ So the jump goes to address 0x1004_0000

🧩 Why This Matters:

- The jump can’t go to any address in the whole 4 GB space only within the same 256 MB block (because we reuse the top 4 bits).

- This saves space in the instruction (only 26 bits needed instead of 32).