## ✅ What is Cache Memory?

Cache memory is a small, very fast memory located close to the CPU.
Its main purpose is:

👉 To store frequently used data and instructions so the CPU can access them quickly.

Because accessing RAM is slower, cache helps speed up the overall performance of the computer.

## ✅ Types of Cache: L1, L2, L3

Modern processors have caches in different levels:

### 🔹 L1 Cache (Level 1 Cache)

- Fastest cache memory.

- Smallest size (typically 32KB–128KB).

- Located inside the CPU core.

- Usually divided into:

  - L1 Instruction Cache (I-Cache) → stores instructions.

  - L1 Data Cache (D-Cache) → stores data.

👉 If the CPU finds data here, access is extremely fast.

### 🔹 L2 Cache (Level 2 Cache)

- Larger than L1 (256KB–1MB).

- Slower than L1, but still faster than RAM.

- Can be inside each CPU core or shared by some cores (depends on architecture).

👉 It acts as a backup for L1 cache.

### 🔹 L3 Cache (Level 3 Cache)

- Largest cache (4MB–64MB).

- Slower than L1 and L2, but faster than RAM.

- Shared by all CPU cores in most processors.

👉 Helps reduce bottlenecks between CPU cores.

## 📌 Why do we need 3 levels of cache?

Because:

- Fast memory is very expensive, so it must be small.

- We use a hierarchy to balance:

  - Speed

  - Cost

  - Storage capacity

So:

➤ CPU checks L1 first → if not found,
➤ Checks L2 → if not found,
➤ Checks L3 → if not found,
➤ Then goes to RAM, which is much slower.

## 📌 Cache Hit vs Cache Miss

- Cache Hit → data found in cache → fast

- Cache Miss → data NOT in cache → must fetch from RAM → slow

Goal: Increase cache hits to improve performance.